(cluster_env) [viktor@n225 Extension_LLM-DES-MOO-Modeling]$ python3 main.py

MachineName  Avg_Time_per_part_s  Availability_%  MTTR_s  EnergyIdling  EnergyWorking

Conveyor belt                  6.0          100.00     1.0          0.00           0.00

Hantering cell                 25.0           97.79    74.0          0.50           0.74

Loading robot                 12.0           90.49    68.0          0.25           0.72

Presses cell 1                175.0           87.79    73.0          1.25           1.28

Presses cell 2                176.0           87.69    74.0          1.25           1.27

Quality station cell                 41.0           85.87    66.0          0.58           0.84

Washing machine                 14.0           80.89   269.0          4.28          35.24

Directly-follows relationships:

Conveyor belt -> Washing machine

Presses cell 2 -> Quality station cell

Washing machine -> Hantering cell

Loading robot -> Conveyor belt

Hantering cell -> Presses cell 1

Parallel relationships:

Presses cell 2 || Presses cell 1

Presses cell 1 || Presses cell 2

Builder activated:

Inspector activated:

Building initial model based on the blueprint and input data...

Model saved to results/initial_model.py

Inspecting initial model for errors...

Inspecting code (Attempt 1)...

Run successful! The code is without errors.

Model saved to results/initial_model.py

Visualizing initial model into flow chart...

Visualizer activated:

Flow chart saved to: results/model_visualization.png

Evaluating initial model...

['----Results from model: Original model', 'Running simulation 10 times...', '', '=== Mean Overall KPIs over 10 runs ===', 'Throughput = 27.23 parts/hour', 'WIP = 18.25 parts', 'Mean Energy Consumption per Part = 0.4541 kWh/part']

=================================

---OPTIONAL OPTIMIZATION STAGE---

=================================

Optimizer activated:

==============================

-OPTIMIZATION INITIALIZED-

---OBJECTIVE SELECTION---

==============================

Please choose one of the following optimization scenarios:

1. WIP (Min) vs. Throughput (Max)

2. WIP (Min) vs. Energy Consumption per part (Min)

3. Throughput (Max) vs. Energy Consumption per part (Min)

[CONFIRMED] Scenario 1 loaded:

-> Objective 1: WIP (MIN)

-> Objective 2: THROUGHPUT (MAX)

------------------------------

================================

---DECISION VARIABLES SELECTION---

================================

Please choose one of the following approaches to optimizing your selected objectives:

1. Find the best buffer configuration

2. Decrease machine process time by som chosen percentage

3. Increase machine availability by some chosen percentage

4. Decrease machine MTTR by some chosen percentage

[CONFIRMED] Scenario 1 loaded:

The MOO algorithm will use these input variables to optimize the objectives:

All buffer capacities. Discrete values on the range 1-10

------------------------------

=================================

---HYPER PARAMETER SELECTION---

=================================

------------------------------

Generating MOO code...

Model saved to results/MOO_initial_code.py

Combining the MOO code with the simulation code...

Model saved to results/initial_combined_code.py

Generating a vizualization of the MOO algorithm...

Inspecting MOO and simulation code (Attempt 1)...

Model saved to results/checked_initial_combined_code.py

Running the MOO algorithm...

Run successful! Time taken: 389.44 seconds

See the results of the MOO algorithm in 'moo_simulation_results.csv'

And the pareto optimal solutions here: 'moo_pareto_solutions.csv'

Generating suggestions for improvements based on the Pareto-optimal solutions and user input...

The suggested datapoints for adaption can be seen in: results/'suggested_improvements.csv'

Suggested changes based on the MOO algorithm

{'instructions': [{'post_loading_cap': 1, 'post_conveyor_cap': 1, 'post_washing_cap': 1, 'pre_press1_cap': 3, 'pre_press2_cap': 2, 'post_press12_cap': 3, 'throughput': 27.25952380952381, 'wip': 14.198214285714286}, {'post_loading_cap': 1, 'post_conveyor_cap': 1, 'post_washing_cap': 1, 'pre_press1_cap': 2, 'pre_press2_cap': 2, 'post_press12_cap': 3, 'throughput': 27.22619047619048, 'wip': 13.199007936507936}, {'post_loading_cap': 1, 'post_conveyor_cap': 1, 'post_washing_cap': 1, 'pre_press1_cap': 2, 'pre_press2_cap': 4, 'post_press12_cap': 1, 'throughput': 27.32857142857143, 'wip': 15.040674603174605}]}

Below I explain each suggested buffer-capacity setting in plain production terms: what to change, why the optimizer recommends it, and the practical trade-offs so you can pick by priority.

Suggestion A — balanced flow (post_loading=1, post_conveyor=1, post_washing=1, pre_press1=3, pre_press2=2, post_press12=3)

- What it does: shrinks the early delay buffers to 1 (so less inventory sits between loading/conveyor/wash) and gives slightly more feed capacity to pre_press1 than pre_press2, with a moderate post-press buffer.

- Why it helps: reducing early-stage buffer capacity prevents large accumulations upstream, which keeps WIP moderate (WIP ≈ 14.2) while maintaining throughput (≈27.26 parts/h). The asymmetric pre-press split keeps the two press lines fed without overfeeding either one, reducing blocking and idle cycles.

- Trade-offs/risks: smaller early buffers increase risk of short-term starvation if upstream variability spikes; but with these press buffer sizes the system stays reasonably robust. Good if you want a compromise between throughput and lower inventory.

Suggestion B — minimum WIP (post_loading=1, post_conveyor=1, post_washing=1, pre_press1=2, pre_press2=2, post_press12=3)

- What it does: same small early buffers but reduces both pre-press buffers to 2 (more symmetry, less intermediate buffering).

- Why it helps: this produces the lowest average WIP of the three (WIP ≈ 13.20) while keeping throughput nearly unchanged (≈27.23 parts/h). Symmetric, smaller pre-press buffers force a tighter, more level flow into the presses so less WIP accumulates.

- Trade-offs/risks: you gain lower inventory and lower holding/waiting energy, but you accept a slightly higher sensitivity to variability—shorter buffers mean the presses are more exposed to upstream hiccups. Choose this if your priority is minimizing WIP/carrying cost.

Suggestion C — throughput-first (post_loading=1, post_conveyor=1, post_washing=1, pre_press1=2, pre_press2=4, post_press12=1)

- What it does: increases pre_press2 capacity (feeds one press more aggressively) and dramatically reduces the post-press buffer to 1.

- Why it helps: pushing more into the presses reduces their idle time and yields the highest measured throughput (≈27.33 parts/h). This setting effectively biases the flow to keep machines busy.

- Trade-offs/risks: WIP is highest here (≈15.04) because items accumulate in and around the presses and get blocked at the tiny post-press buffer; blocking can increase blocked_time and waiting energy. This is the choice when maximizing output is top priority and you can tolerate more in-process inventory and potential blocking.

Practical next steps (short)

- Pick the suggestion matching your priority: B for lowest WIP, C for max throughput, A for middle ground.

- Implement incrementally (change one buffer at a time), monitor throughput, WIP, and machine blocking/starvation metrics, and be ready to revert if variability or breakdowns increase starvation.

back in main.py

Safety Inspector activated:

1st change:

{'post_loading_cap': 1, 'post_conveyor_cap': 1, 'post_washing_cap': 1, 'pre_press1_cap': 3, 'pre_press2_cap': 2, 'post_press12_cap': 3, 'throughput': 27.25952380952381, 'wip': 14.198214285714286}

Why critical:

- Several buffer capacities differ from the required originals:

- PostLoadingBuffer: original 2 -> changed to 1

- PostConveyorBuffer: original 2 -> changed to 1

- PostWashingBuffer: original 2 -> changed to 1

- PrePress2Buffer: original 3 -> changed to 2

These violate the rule that all buffer capacities must be kept at their original levels, so this change is critical.

2nd change:

{'post_loading_cap': 1, 'post_conveyor_cap': 1, 'post_washing_cap': 1, 'pre_press1_cap': 2, 'pre_press2_cap': 2, 'post_press12_cap': 3, 'throughput': 27.22619047619048, 'wip': 13.199007936507936}

Why critical:

- Multiple buffer capacities differ from originals:

- PostLoadingBuffer: 2 -> 1

- PostConveyorBuffer: 2 -> 1

- PostWashingBuffer: 2 -> 1

- PrePress1Buffer: 3 -> 2

- PrePress2Buffer: 3 -> 2

These also violate the buffer-capacity requirement, so this change is critical.

3rd change:

{'post_loading_cap': 1, 'post_conveyor_cap': 1, 'post_washing_cap': 1, 'pre_press1_cap': 2, 'pre_press2_cap': 4, 'post_press12_cap': 1, 'throughput': 27.32857142857143, 'wip': 15.040674603174605}

Why critical:

- Several buffer capacities differ from originals:

- PostLoadingBuffer: 2 -> 1

- PostConveyorBuffer: 2 -> 1

- PostWashingBuffer: 2 -> 1

- PrePress1Buffer: 3 -> 2

- PrePress2Buffer: 3 -> 4

- PostPress1&Press2Buffer: 3 -> 1

All of these are changes to required buffer capacities, so this change is critical.

Note: The press processtime requirement (>= 60 s) cannot be checked because none of the suggested changes include press processtime.

Adapting process:

Step 1 Worker activated:

Instruction for model adaptation: {'post_loading_cap': 1, 'post_conveyor_cap': 1, 'post_washing_cap': 1, 'pre_press1_cap': 3, 'pre_press2_cap': 2, 'post_press12_cap': 3, 'throughput': 27.25952380952381, 'wip': 14.198214285714286}

Model saved to results/Adapted_model_version_1.py

['----Results from model: Adapted_model_version 1', 'Running simulation 10 times...', '', '=== Mean Overall KPIs over 10 runs ===', 'Throughput = 27.11 parts/hour', 'WIP = 14.23 parts', 'Mean Energy Consumption per Part = 0.4548 kWh/part']

Adapting process:

Step 2 Worker activated:

Instruction for model adaptation: {'post_loading_cap': 1, 'post_conveyor_cap': 1, 'post_washing_cap': 1, 'pre_press1_cap': 2, 'pre_press2_cap': 2, 'post_press12_cap': 3, 'throughput': 27.22619047619048, 'wip': 13.199007936507936}

Model saved to results/Adapted_model_version_2.py

['----Results from model: Adapted_model_version 2', 'Running simulation 10 times...', '', '=== Mean Overall KPIs over 10 runs ===', 'Throughput = 26.98 parts/hour', 'WIP = 13.21 parts', 'Mean Energy Consumption per Part = 0.4563 kWh/part']

Adapting process:

Step 3 Worker activated:

Instruction for model adaptation: {'post_loading_cap': 1, 'post_conveyor_cap': 1, 'post_washing_cap': 1, 'pre_press1_cap': 2, 'pre_press2_cap': 4, 'post_press12_cap': 1, 'throughput': 27.32857142857143, 'wip': 15.040674603174605}

Model saved to results/Adapted_model_version_3.py

['----Results from model: Adapted_model_version 3', 'Running simulation 10 times...', '', '=== Mean Overall KPIs over 10 runs ===', 'Throughput = 27.14 parts/hour', 'WIP = 15.02 parts', 'Mean Energy Consumption per Part = 0.4545 kWh/part']

Evaluator activated:

Summary of results (key KPIs):

- Original

- Throughput = 27.23 parts/hour

- WIP = 18.25 parts

- Energy/part = 0.4541 kWh

- Adapted_model_version 1

- Throughput = 27.11 parts/hour

- WIP = 14.23 parts

- Energy/part = 0.4548 kWh

- Adapted_model_version 2

- Throughput = 26.98 parts/hour

- WIP = 13.21 parts

- Energy/part = 0.4563 kWh

- Adapted_model_version 3

- Throughput = 27.14 parts/hour

- WIP = 15.02 parts

- Energy/part = 0.4545 kWh

Recommendation and rationale

- Best overall trade-off: Adapted_model_version 3.

- It cuts WIP by ~17.7% versus Original (18.25 → 15.02) while causing only a tiny throughput drop (~0.33%, 27.23 → 27.14) and a negligible increase in energy per part (+0.0004 kWh, ~0.09%).

- Compared with the other adapted versions, V3 achieves a good middle ground: substantial WIP reduction with minimal penalties.

- If your priority is maximizing throughput and minimizing energy/part: keep the Original model (highest throughput 27.23 and lowest energy 0.4541 kWh/part), at the cost of higher WIP.

- If your priority is minimizing WIP regardless of small throughput/energy penalties: Adapted_model_version 2 gives the lowest WIP (13.21, ~27.6% reduction vs Original) but incurs the largest throughput drop (~0.9%) and slightly higher energy/part.

Suggested next steps

- Confirm these conclusions with statistical testing or more runs (to get confidence intervals) if decisions are critical.

- If you want, I can compute percent changes, plot the trade-offs, or recommend which configuration to deploy based on your objective (throughput vs WIP vs energy).

Figure saved to results/model_comparison_kpis.png

Total execution time: 1113.01 seconds