(cluster_env) [viktor@n213 Extension_LLM-DES-MOO-Modeling]$ python3 main.py

MachineName  Avg_Time_per_part_s  Availability_%  MTTR_s  EnergyIdling  EnergyWorking

Conveyor belt                  6.0          100.00     1.0          0.00           0.00

Hantering cell                 25.0           97.79    74.0          0.50           0.74

Loading robot                 12.0           90.49    68.0          0.25           0.72

Presses cell 1                175.0           87.79    73.0          1.25           1.28

Presses cell 2                176.0           87.69    74.0          1.25           1.27

Quality station cell                 41.0           85.87    66.0          0.58           0.84

Washing machine                 14.0           80.89   269.0          4.28          35.24

Directly-follows relationships:

Conveyor belt -> Washing machine

Presses cell 2 -> Quality station cell

Loading robot -> Conveyor belt

Washing machine -> Hantering cell

Hantering cell -> Presses cell 1

Parallel relationships:

Presses cell 1 || Presses cell 2

Presses cell 2 || Presses cell 1

Builder activated:

Inspector activated:

Building initial model based on the blueprint and input data...

Model saved to results/initial_model.py

Inspecting initial model for errors...

Inspecting code (Attempt 1)...

Run successful! The code is without errors.

Model saved to results/initial_model.py

Visualizing initial model into flow chart...

Visualizer activated:

Flow chart saved to: results/model_visualization.png

Evaluating initial model...

['----Results from model: Original model', 'Running simulation 10 times...', '', '=== Mean Overall KPIs over 10 runs ===', 'Throughput = 27.10 parts/hour', 'WIP = 17.10 parts', 'Mean Energy Consumption per Part = 0.4543 kWh/part']

=================================

---OPTIONAL OPTIMIZATION STAGE---

=================================

Optimizer activated:

==============================

-OPTIMIZATION INITIALIZED-

---OBJECTIVE SELECTION---

==============================

Please choose one of the following optimization scenarios:

1. WIP (Min) vs. Throughput (Max)

2. WIP (Min) vs. Energy Consumption per part (Min)

3. Throughput (Max) vs. Energy Consumption per part (Min)

[CONFIRMED] Scenario 1 loaded:

-> Objective 1: WIP (MIN)

-> Objective 2: THROUGHPUT (MAX)

------------------------------

================================

---DECISION VARIABLES SELECTION---

================================

Please choose one of the following approaches to optimizing your selected objectives:

1. Find the best buffer configuration

2. Decrease machine process time by som chosen percentage

3. Increase machine availability by some chosen percentage

4. Decrease machine MTTR by some chosen percentage

[CONFIRMED] Scenario 1 loaded:

The MOO algorithm will use these input variables to optimize the objectives:

All buffer capacities. Discrete values on the range 1-10

------------------------------

=================================

---HYPER PARAMETER SELECTION---

=================================

------------------------------

Generating MOO code...

Model saved to results/MOO_initial_code.py

Combining the MOO code with the simulation code...

Model saved to results/initial_combined_code.py

Generating a vizualization of the MOO algorithm...

Inspecting MOO and simulation code (Attempt 1)...

Model saved to results/checked_initial_combined_code.py

Running the MOO algorithm...

Run successful! Time taken: 270.55 seconds

See the results of the MOO algorithm in 'moo_simulation_results.csv'

And the pareto optimal solutions here: 'moo_pareto_solutions.csv'

Generating suggestions for improvements based on the Pareto-optimal solutions and user input...

The suggested datapoints for adaption can be seen in: results/'suggested_improvements.csv'

Suggested changes based on the MOO algorithm

{'instructions': [{'buffer1_cap': 1, 'buffer2_cap': 1, 'buffer3_cap': 1, 'buffer4_cap': 2, 'buffer5_cap': 1, 'buffer6_cap': 1, 'wip': 11.689484126984128, 'throughput': 26.486904761904764}, {'buffer1_cap': 1, 'buffer2_cap': 1, 'buffer3_cap': 1, 'buffer4_cap': 8, 'buffer5_cap': 2, 'buffer6_cap': 1, 'wip': 15.499206349206348, 'throughput': 27.057142857142857}, {'buffer1_cap': 5, 'buffer2_cap': 1, 'buffer3_cap': 8, 'buffer4_cap': 9, 'buffer5_cap': 8, 'buffer6_cap': 2, 'wip': 33.523809523809526, 'throughput': 27.40714285714286}]}

Summary: each suggestion is a specific set of buffer capacities (post-loading, post-conveyor, post-washing, pre-press1, pre-press2, post-press12). The MOO results show a clear trade‑off: smaller buffers → low WIP but lower throughput; larger buffers → higher throughput but more WIP. Pick the option that matches your priority.

Suggestion 1 — very small buffers (cap: post_loading=1, post_conveyor=1, post_washing=1, pre_press1=2, pre_press2=1, post_press12=1)

- Result: WIP ≈ 11.7 parts, Throughput ≈ 26.49 parts/hour.

- What changed: almost every delay buffer is reduced to the minimum.

- Why it helps: small buffers keep inventory low and reduce in‑system holding and energy tied up in WIP. You’ll see shorter average queues and lower capital/space exposure.

- Trade‑off / operational effect: presses and downstream machines will be more tightly coupled to upstream availability, so occasional blocking or starvation will reduce peak throughput slightly. Choose this if minimizing WIP (floor space, inventory cost) is the top priority.

Suggestion 2 — focused decoupling before presses (cap: post_loading=1, post_conveyor=1, post_washing=1, pre_press1=8, pre_press2=2, post_press12=1)

- Result: WIP ≈ 15.5 parts, Throughput ≈ 27.06 parts/hour.

- What changed: you keep most buffers small but substantially enlarge the pre-press buffers (especially pre_press1).

- Why it helps: the presses are long processing steps and sensitive to upstream variability. Bigger pre-press buffers let presses run more continuously (less starvation), which raises throughput while only moderately increasing WIP. This targets the main bottleneck (the press stage) with limited additional inventory.

- Trade‑off / operational effect: modest increase in WIP for a noticeable throughput gain — a practical compromise if you want better throughput without high inventory.

Suggestion 3 — large decoupling across the line (cap: post_loading=5, post_conveyor=1, post_washing=8, pre_press1=9, pre_press2=8, post_press12=2)

- Result: WIP ≈ 33.5 parts, Throughput ≈ 27.41 parts/hour (highest).

- What changed: many buffers are enlarged, especially around washing and the two pre-press feeds.

- Why it helps: large buffers decouple almost every stage so machines rarely starve or block. That maximizes utilization and delivers the highest throughput from the simulation.

- Trade‑off / operational effect: substantially higher WIP (more inventory in transit and stored), higher capital/space requirements and potentially more energy/handling per part. Choose this only if throughput is the overriding KPI and you can accept the inventory cost.

Short recommendation:

- If your priority is lowest WIP (space, inventory cost) pick Suggestion 1.

- If you want a balanced, cost‑efficient throughput gain, pick Suggestion 2 (best practical trade‑off).

- If throughput is the single priority and inventory cost is acceptable, pick Suggestion 3.

back in main.py

Safety Inspector activated:

Assuming buffer1_cap..buffer6_cap map to the named buffers in order:

- buffer1_cap = PostLoadingBuffer (orig 2)

- buffer2_cap = PostConveyorBuffer (orig 2)

- buffer3_cap = PostWashingBuffer (orig 2)

- buffer4_cap = PrePress1Buffer (orig 3)

- buffer5_cap = PrePress2Buffer (orig 3)

- buffer6_cap = PostPress1&Press2Buffer (orig 3)

Original capacities: [2, 2, 2, 3, 3, 3].

All three suggested changes modify one or more of these buffer capacities, so they are critical:

1st change:

{'buffer1_cap': 1, 'buffer2_cap': 1, 'buffer3_cap': 1, 'buffer4_cap': 2, 'buffer5_cap': 1, 'buffer6_cap': 1, 'wip': 11.689484126984128, 'throughput': 26.486904761904764}

- Deviations from original:

- PostLoadingBuffer: 2 -> 1

- PostConveyorBuffer: 2 -> 1

- PostWashingBuffer: 2 -> 1

- PrePress1Buffer: 3 -> 2

- PrePress2Buffer: 3 -> 1

- PostPress1&Press2Buffer: 3 -> 1

- Why critical: it changes all required buffer capacities (violates the rule that all buffers must remain at original levels).

2nd change:

{'buffer1_cap': 1, 'buffer2_cap': 1, 'buffer3_cap': 1, 'buffer4_cap': 8, 'buffer5_cap': 2, 'buffer6_cap': 1, 'wip': 15.499206349206348, 'throughput': 27.057142857142857}

- Deviations:

- PostLoadingBuffer: 2 -> 1

- PostConveyorBuffer: 2 -> 1

- PostWashingBuffer: 2 -> 1

- PrePress1Buffer: 3 -> 8

- PrePress2Buffer: 3 -> 2

- PostPress1&Press2Buffer: 3 -> 1

- Why critical: multiple buffer capacities changed from their originals.

3rd change:

{'buffer1_cap': 5, 'buffer2_cap': 1, 'buffer3_cap': 8, 'buffer4_cap': 9, 'buffer5_cap': 8, 'buffer6_cap': 2, 'wip': 33.523809523809526, 'throughput': 27.40714285714286}

- Deviations:

- PostLoadingBuffer: 2 -> 5

- PostConveyorBuffer: 2 -> 1

- PostWashingBuffer: 2 -> 8

- PrePress1Buffer: 3 -> 9

- PrePress2Buffer: 3 -> 8

- PostPress1&Press2Buffer: 3 -> 2

- Why critical: again, all buffer capacities differ from their original values.

Note: none of the changes include press processtime, so I cannot check the "presses need processtime >= 60s" constraint; but because the buffer capacities are altered in all three suggestions, each is a critical change.

Adapting process:

Step 1 Worker activated:

Instruction for model adaptation: {'buffer1_cap': 1, 'buffer2_cap': 1, 'buffer3_cap': 1, 'buffer4_cap': 2, 'buffer5_cap': 1, 'buffer6_cap': 1, 'wip': 11.689484126984128, 'throughput': 26.486904761904764}

Model saved to results/Adapted_model_version_1.py

['----Results from model: Adapted_model_version 1', 'Running simulation 10 times...', '', '=== Mean Overall KPIs over 10 runs ===', 'Throughput = 26.56 parts/hour', 'WIP = 11.73 parts', 'Mean Energy Consumption per Part = 0.4615 kWh/part']

Adapting process:

Step 2 Worker activated:

Instruction for model adaptation: {'buffer1_cap': 1, 'buffer2_cap': 1, 'buffer3_cap': 1, 'buffer4_cap': 8, 'buffer5_cap': 2, 'buffer6_cap': 1, 'wip': 15.499206349206348, 'throughput': 27.057142857142857}

Model saved to results/Adapted_model_version_2.py

['----Results from model: Adapted_model_version 2', 'Running simulation 10 times...', '', '=== Mean Overall KPIs over 10 runs ===', 'Throughput = 27.05 parts/hour', 'WIP = 15.38 parts', 'Mean Energy Consumption per Part = 0.4553 kWh/part']

Adapting process:

Step 3 Worker activated:

Instruction for model adaptation: {'buffer1_cap': 5, 'buffer2_cap': 1, 'buffer3_cap': 8, 'buffer4_cap': 9, 'buffer5_cap': 8, 'buffer6_cap': 2, 'wip': 33.523809523809526, 'throughput': 27.40714285714286}

Model saved to results/Adapted_model_version_3.py

['----Results from model: Adapted_model_version 3', 'Running simulation 10 times...', '', '=== Mean Overall KPIs over 10 runs ===', 'Throughput = 27.22 parts/hour', 'WIP = 32.70 parts', 'Mean Energy Consumption per Part = 0.4544 kWh/part']

Evaluator activated:

Summary of results (10-run means)

Metrics: Throughput (higher better), WIP (lower better), Energy per part (lower better).

- Original model

- Throughput = 27.10 parts/h

- WIP = 17.10 parts

- Energy = 0.4543 kWh/part

- Adapted_model_version 1

- Throughput = 26.56 parts/h (−0.54, −2.0% vs original)

- WIP = 11.73 parts (−5.37, −31.4%)

- Energy = 0.4615 kWh/part (+0.0072, +1.6%)

- Adapted_model_version 2

- Throughput = 27.05 parts/h (−0.05, −0.18%)

- WIP = 15.38 parts (−1.72, −10.1%)

- Energy = 0.4553 kWh/part (+0.0010, +0.22%)

- Adapted_model_version 3

- Throughput = 27.22 parts/h (+0.12, +0.44%)

- WIP = 32.70 parts (+15.60, +91.2%)

- Energy = 0.4544 kWh/part (+0.0001, +0.02%)

Which configuration is “best”

- If you need the best throughput: Adapted_model_version 3 wins (27.22), but it comes at a very large WIP penalty (≈+91%).

- If you need the smallest WIP (inventory, floor space, lead-time): Adapted_model_version 1 wins (11.73) but with lower throughput (≈−2%) and higher energy (+1.6%).

- If you need the lowest energy per part: the Original is marginally best (0.4543), but differences across models are very small (<~1.6%).

- Best overall trade-off: Adapted_model_version 2. It preserves throughput almost exactly (−0.18%), reduces WIP moderately (−10%), and has negligible extra energy (+0.22%). That makes it the most balanced choice across all three KPIs.

Recommendations / next steps

- If you care about a balanced improvement, choose Adapted_model_version 2.

- If one KPI is dominant for your business (throughput vs WIP vs energy), choose the model that matches that priority (v3 for throughput, v1 for WIP, Original for minimal energy).

- Because the runs are stochastic (only 10 runs), I recommend more runs and a simple statistical check (means ± std, t-test or confidence intervals) to confirm differences are significant before deploying changes.

- If you want, I can:

- compute percentage changes and ranks in a table,

- run a Pareto/multi-objective selection or a weighted-score ranking if you give KPI weights,

- or suggest experiment sizes to reach desired statistical confidence.

Figure saved to results/model_comparison_kpis.png

Total execution time: 907.03 seconds