(cluster_env) [arvkru@n243 Extension_LLM-DES-MOO-Modeling]$ python3 main.py

MachineName  Avg_Time_per_part_s  Availability_%  MTTR_s  EnergyIdling  EnergyWorking

Conveyor belt                  6.0          100.00     1.0          0.00           0.00

Hantering cell                 25.0           97.79    74.0          0.50           0.74

Loading robot                 12.0           90.49    68.0          0.25           0.72

Presses cell 1                175.0           87.79    73.0          1.25           1.28

Presses cell 2                176.0           87.69    74.0          1.25           1.27

Quality station cell                 41.0           85.87    66.0          0.58           0.84

Washing machine                 14.0           80.89   269.0          4.28          35.24

Directly-follows relationships:

Presses cell 2 -> Quality station cell

Hantering cell -> Presses cell 1

Conveyor belt -> Washing machine

Loading robot -> Conveyor belt

Washing machine -> Hantering cell

Parallel relationships:

Presses cell 1 || Presses cell 2

Presses cell 2 || Presses cell 1

Builder activated:

Building initial model based on the blueprint and input data...

Model saved to results/initial_model.py

Visualizing initial model into flow chart...

Visualizer activated:

Flow chart saved to: results/model_visualization.png

Evaluating initial model...

['----Results from model: Original model', '', '=== Mean Overall KPIs over 10 runs ===', 'Throughput = 27.34 parts/hour', 'WIP = 18.16 parts', 'Mean Energy Consumption per Part = 0.4525 kWh/part']

=================================

---OPTIONAL OPTIMIZATION STAGE---

=================================

Optimizer activated:

==============================

-OPTIMIZATION INITIALIZED-

---OBJECTIVE SELECTION---

==============================

Please choose one of the following optimization scenarios:

1. WIP (Min) vs. Throughput (Max)

2. WIP (Min) vs. Energy Consumption per part (Min)

3. Throughput (Max) vs. Energy Consumption per part (Min)

[CONFIRMED] Scenario 1 loaded:

-> Objective 1: WIP (MIN)

-> Objective 2: THROUGHPUT (MAX)

------------------------------

================================

---DECISION VARIABLES SELECTION---

================================

Please choose one of the following approaches to optimizing your selected objectives:

1. Find the best buffer configuration

2. Decrease machine process time by som chosen percentage

3. Increase machine availability by some chosen percentage

4. Decrease machine MTTR by some chosen percentage

[CONFIRMED] Scenario 1 loaded:

The MOO algorithm will use these input variables to optimize the objectives:

All buffer capacities. Discrete values on the range 1-10

------------------------------

=================================

---HYPER PARAMETER SELECTION---

=================================

------------------------------

Generating MOO code...

Model saved to results/MOO_initial_code.py

Combining the MOO code with the simulation code...

Model saved to results/initial_combined_code.py

Generating a vizualization of the MOO algorithm...

Model saved to results/checked_initial_combined_code.py

Running the MOO algorithm...

Run successful! Time taken: 397.03 seconds

See the results of the MOO algorithm in 'moo_simulation_results.csv'

And the pareto optimal solutions here: 'moo_pareto_solutions.csv'

Generating suggestions for improvements based on the Pareto-optimal solutions and user input...

The suggested datapoints for adaption can be seen in: results/'suggested_improvements.csv'

Suggested changes based on the MOO algorithm

{'instructions': [{'cap_post_loading_buffer': 1, 'cap_post_conveyor_buffer': 1, 'cap_post_washing_buffer': 1, 'cap_pre_press1_buffer': 2, 'cap_pre_press2_buffer': 2, 'cap_post_press12_buffer': 6, 'wip': 13.166666666666668, 'throughput': 27.19761904761905}, {'cap_post_loading_buffer': 1, 'cap_post_conveyor_buffer': 1, 'cap_post_washing_buffer': 1, 'cap_pre_press1_buffer': 2, 'cap_pre_press2_buffer': 2, 'cap_post_press12_buffer': 8, 'wip': 13.249206349206348, 'throughput': 27.226190476190474}, {'cap_post_loading_buffer': 1, 'cap_post_conveyor_buffer': 1, 'cap_post_washing_buffer': 2, 'cap_pre_press1_buffer': 2, 'cap_pre_press2_buffer': 2, 'cap_post_press12_buffer': 6, 'wip': 14.103373015873016, 'throughput': 27.276190476190475}]}

Suggestion 1

- Changes (from current setup: post_loading=2, post_conveyor=2, post_washing=2, pre_press1=3, pre_press2=3, post_press12=3):

- post_loading_buffer -> 1

- post_conveyor_buffer -> 1

- post_washing_buffer -> 1

- pre_press1_buffer -> 2

- pre_press2_buffer -> 2

- post_press12_buffer -> 6

- MOO outcome: WIP ≈ 13.17 parts, Throughput ≈ 27.20 parts/hour

- Why this helps:

- Tightening upstream buffers (loading → washing and the pre-press split) reduces work-in-progress by preventing large buildups before the presses. That lowers inventory holding and the amount of material circulating through the line.

- Increasing the post-press merge buffer (post_press12 to 6) decouples the two parallel presses from the downstream quality station. This reduces blocking at the presses (they can finish parts even if the quality station is momentarily busy or down), which preserves or recovers throughput while keeping WIP low.

- Practical note: This configuration deliberately trades off small upstream decoupling for a larger downstream buffer to keep throughput stable while minimizing WIP.

Suggestion 2

- Changes:

- same as Suggestion 1 except post_press12_buffer -> 8

- MOO outcome: WIP ≈ 13.25 parts, Throughput ≈ 27.23 parts/hour

- Why this helps:

- Same logic as Suggestion 1, but a slightly larger post-press buffer (8) gives even more decoupling after the presses. That further reduces blocking risk at the presses and slightly improves throughput (very small gain here) at the cost of a negligible increase in average WIP.

- Practical note: Use this if you want to prioritize a tiny throughput improvement and are comfortable holding a few more parts in the merged buffer.

Suggestion 3

- Changes:

- post_loading_buffer -> 1

- post_conveyor_buffer -> 1

- post_washing_buffer -> 2 (kept one of the upstream buffers larger)

- pre_press1_buffer -> 2

- pre_press2_buffer -> 2

- post_press12_buffer -> 6

- MOO outcome: WIP ≈ 14.10 parts, Throughput ≈ 27.28 parts/hour

- Why this helps:

- Keeping the post_washing buffer at 2 (instead of 1) provides slightly more decoupling between the washing step and the hantering/splitter. That reduces starvation at the presses and slightly increases throughput compared with tightening all upstream buffers to 1.

- The downstream post_press12 buffer at 6 preserves the blocking reduction benefit so throughput improves, but the slightly larger upstream buffer increases WIP compared with Suggestion 1.

- Practical note: This balances higher throughput (small gain) against a small increase in WIP — choose it if washing variability or frequent short outages make starvation a concern.

Overall takeaway (short)

- Reducing upstream buffer capacities cuts WIP by preventing large queues.

- Increasing the merged buffer after the parallel presses prevents blocking of those long-processing units and preserves (or slightly raises) throughput.

- Tradeoffs: tighter upstream buffers risk more starvation/interruptions; larger downstream buffer increases WIP but protects throughput. Choose the variant that matches your tolerance for WIP vs. the need for maximum stable throughput.

back in main.py

Safety Inspector activated:

1st change:

{'cap_post_loading_buffer': 1, 'cap_post_conveyor_buffer': 1, 'cap_post_washing_buffer': 1, 'cap_pre_press1_buffer': 2, 'cap_pre_press2_buffer': 2, 'cap_post_press12_buffer': 6, 'wip': 13.166666666666668, 'throughput': 27.19761904761905}

Why critical:

- Several buffer capacities are altered from the required originals:

- PostLoadingBuffer: original 2 -> 1

- PostConveyorBuffer: original 2 -> 1

- PostWashingBuffer: original 2 -> 1

- PrePress1Buffer: original 3 -> 2

- PrePress2Buffer: original 3 -> 2

- PostPress1&Press2Buffer: original 3 -> 6

- The critical rule states all buffer capacities must be kept at their original levels; these changes violate that rule.

2nd change:

{'cap_post_loading_buffer': 1, 'cap_post_conveyor_buffer': 1, 'cap_post_washing_buffer': 1, 'cap_pre_press1_buffer': 2, 'cap_pre_press2_buffer': 2, 'cap_post_press12_buffer': 8, 'wip': 13.249206349206348, 'throughput': 27.226190476190474}

Why critical:

- Same buffer deviations as above (with PostPress1&2 buffer changed to 8 instead of 3), violating the requirement to keep original buffer capacities.

3rd change:

{'cap_post_loading_buffer': 1, 'cap_post_conveyor_buffer': 1, 'cap_post_washing_buffer': 2, 'cap_pre_press1_buffer': 2, 'cap_pre_press2_buffer': 2, 'cap_post_press12_buffer': 6, 'wip': 14.103373015873016, 'throughput': 27.276190476190475}

Why critical:

- Several buffers changed from their originals:

- PostLoadingBuffer: 2 -> 1

- PostConveyorBuffer: 2 -> 1

- PrePress1Buffer: 3 -> 2

- PrePress2Buffer: 3 -> 2

- PostPress1&Press2Buffer: 3 -> 6

- PostWashingBuffer remains at 2 (unchanged), but other changes still violate the buffer-capacity requirement.

Note: None of the changes specify press processtime, so the "processtime >= 60s" rule is not directly affected by these entries.

Adapting process:

Step 1 Worker activated:

Instruction for model adaptation: {'cap_post_loading_buffer': 1, 'cap_post_conveyor_buffer': 1, 'cap_post_washing_buffer': 1, 'cap_pre_press1_buffer': 2, 'cap_pre_press2_buffer': 2, 'cap_post_press12_buffer': 6, 'wip': 13.166666666666668, 'throughput': 27.19761904761905}

Model saved to results/Adapted_model_version_1.py

['----Results from model: Adapted_model_version 1', '', '=== Mean Overall KPIs over 10 runs ===', 'Throughput = 27.18 parts/hour', 'WIP = 13.27 parts', 'Mean Energy Consumption per Part = 0.4546 kWh/part']

Adapting process:

Step 2 Worker activated:

Instruction for model adaptation: {'cap_post_loading_buffer': 1, 'cap_post_conveyor_buffer': 1, 'cap_post_washing_buffer': 1, 'cap_pre_press1_buffer': 2, 'cap_pre_press2_buffer': 2, 'cap_post_press12_buffer': 8, 'wip': 13.249206349206348, 'throughput': 27.226190476190474}

Model saved to results/Adapted_model_version_2.py

['----Results from model: Adapted_model_version 2', '', '=== Mean Overall KPIs over 10 runs ===', 'Throughput = 27.18 parts/hour', 'WIP = 13.27 parts', 'Mean Energy Consumption per Part = 0.4546 kWh/part']

Adapting process:

Step 3 Worker activated:

Instruction for model adaptation: {'cap_post_loading_buffer': 1, 'cap_post_conveyor_buffer': 1, 'cap_post_washing_buffer': 2, 'cap_pre_press1_buffer': 2, 'cap_pre_press2_buffer': 2, 'cap_post_press12_buffer': 6, 'wip': 14.103373015873016, 'throughput': 27.276190476190475}

Model saved to results/Adapted_model_version_3.py

['----Results from model: Adapted_model_version 3', '', '=== Mean Overall KPIs over 10 runs ===', 'Throughput = 27.24 parts/hour', 'WIP = 14.15 parts', 'Mean Energy Consumption per Part = 0.4520 kWh/part']

Evaluator activated:

Summary of results (mean over 10 runs):

- Original

- Throughput: 27.34 parts/hour

- WIP: 18.16 parts

- Energy/part: 0.4525 kWh/part

- Adapted_model_version 1

- Throughput: 27.18 parts/hour (−0.59% vs original)

- WIP: 13.27 parts (−26.9% vs original)

- Energy/part: 0.4546 kWh/part (+0.46% vs original)

- Adapted_model_version 2

- Throughput: 27.18 parts/hour (same as v1)

- WIP: 13.27 parts (same as v1)

- Energy/part: 0.4546 kWh/part (same as v1)

- Note: v2 is identical to v1 in these reported KPIs.

- Adapted_model_version 3

- Throughput: 27.24 parts/hour (−0.37% vs original)

- WIP: 14.15 parts (−22.1% vs original)

- Energy/part: 0.4520 kWh/part (−0.11% vs original)

Recommendation

- Best overall balance: Adapted_model_version 3. It retains virtually the same throughput as the original (only ~0.4% lower), cuts WIP by ~22%, and slightly improves energy per part. This makes v3 the best trade-off across all three KPIs.

- If your priority is strictly minimizing WIP and you can accept a tiny throughput loss and a small energy increase, choose Adapted_model_version 1 (or 2—they are equivalent here).

- If throughput is the sole priority, the Original model is marginally best (very small advantage).

Suggested next steps

- If decisions depend on small KPI differences, run more replications and compute confidence intervals or statistical tests to check significance.

- If you need an explicit trade-off, set weights for throughput/WIP/energy and compute a composite score to pick the optimal configuration.

Figure saved to results/model_comparison_kpis.png

Total execution time: 777.64 seconds