(cluster_env) [arvkru@n243 Extension_LLM-DES-MOO-Modeling]$ python3 main.py

MachineName  Avg_Time_per_part_s  Availability_%  MTTR_s  EnergyIdling  EnergyWorking

Conveyor belt                  6.0          100.00     1.0          0.00           0.00

Hantering cell                 25.0           97.79    74.0          0.50           0.74

Loading robot                 12.0           90.49    68.0          0.25           0.72

Presses cell 1                175.0           87.79    73.0          1.25           1.28

Presses cell 2                176.0           87.69    74.0          1.25           1.27

Quality station cell                 41.0           85.87    66.0          0.58           0.84

Washing machine                 14.0           80.89   269.0          4.28          35.24

Directly-follows relationships:

Conveyor belt -> Washing machine

Hantering cell -> Presses cell 1

Loading robot -> Conveyor belt

Washing machine -> Hantering cell

Presses cell 2 -> Quality station cell

Parallel relationships:

Presses cell 2 || Presses cell 1

Presses cell 1 || Presses cell 2

Builder activated:

Building initial model based on the blueprint and input data...

Model saved to results/initial_model.py

Visualizing initial model into flow chart...

Visualizer activated:

Flow chart saved to: results/model_visualization.png

Evaluating initial model...

['----Results from model: Original model', 'Running simulation 10 times...', '', '=== Mean Overall KPIs over 10 runs ===', 'Throughput = 27.35 parts/hour', 'WIP = 18.18 parts', 'Mean Energy Consumption per Part = 0.4522 kWh/part']

=================================

---OPTIONAL OPTIMIZATION STAGE---

=================================

Optimizer activated:

==============================

-OPTIMIZATION INITIALIZED-

---OBJECTIVE SELECTION---

==============================

Please choose one of the following optimization scenarios:

1. WIP (Min) vs. Throughput (Max)

2. WIP (Min) vs. Energy Consumption per part (Min)

3. Throughput (Max) vs. Energy Consumption per part (Min)

[CONFIRMED] Scenario 1 loaded:

-> Objective 1: WIP (MIN)

-> Objective 2: THROUGHPUT (MAX)

------------------------------

================================

---DECISION VARIABLES SELECTION---

================================

Please choose one of the following approaches to optimizing your selected objectives:

1. Find the best buffer configuration

2. Decrease machine process time by som chosen percentage

3. Increase machine availability by some chosen percentage

4. Decrease machine MTTR by some chosen percentage

[CONFIRMED] Scenario 1 loaded:

The MOO algorithm will use these input variables to optimize the objectives:

All buffer capacities. Discrete values on the range 1-10

------------------------------

=================================

---HYPER PARAMETER SELECTION---

=================================

------------------------------

Generating MOO code...

Model saved to results/MOO_initial_code.py

Combining the MOO code with the simulation code...

Model saved to results/initial_combined_code.py

Generating a vizualization of the MOO algorithm...

Model saved to results/checked_initial_combined_code.py

Running the MOO algorithm...

Run successful! Time taken: 347.86 seconds

See the results of the MOO algorithm in 'moo_simulation_results.csv'

And the pareto optimal solutions here: 'moo_pareto_solutions.csv'

Generating suggestions for improvements based on the Pareto-optimal solutions and user input...

The suggested datapoints for adaption can be seen in: results/'suggested_improvements.csv'

Suggested changes based on the MOO algorithm

{'instructions': [{'cap_post_loading': 1, 'cap_post_conveyor': 1, 'cap_post_washing': 1, 'cap_pre_press1': 3, 'cap_pre_press2': 2, 'cap_post_press12': 9, 'wip': 14.291666666666666, 'throughput': 27.39047619047619}, {'cap_post_loading': 1, 'cap_post_conveyor': 1, 'cap_post_washing': 1, 'cap_pre_press1': 2, 'cap_pre_press2': 2, 'cap_post_press12': 3, 'wip': 13.279960317460317, 'throughput': 27.30238095238095}, {'cap_post_loading': 1, 'cap_post_conveyor': 2, 'cap_post_washing': 1, 'cap_pre_press1': 3, 'cap_pre_press2': 2, 'cap_post_press12': 8, 'wip': 15.251587301587302, 'throughput': 27.43452380952381}]}

Summary: each suggestion is a different buffer-capacity configuration found by the MOO run. All three keep throughput roughly the same (~27.3 parts/h) while trading off WIP (work‑in‑process). Below I explain, in plain terms, what each change does, why it helps, and the main trade-offs so you can pick based on whether you care more about low WIP (space/energy) or slightly higher throughput.

Suggestion A (cap_post_loading=1, cap_post_conveyor=1, cap_post_washing=1, cap_pre_press1=3, cap_pre_press2=2, cap_post_press12=9)

- What it changes: it reduces the three early delay buffers (post-loading, post-conveyor, post-washing) to 1 item each and slightly reduces pre-press2, while greatly increasing the buffer after the two presses (post_press12) to 9.

- Why it helps: small upstream buffers prevent long queues building ahead of the slow/less-available machines (washing, conveyor) so overall WIP is lower. The large post-press buffer allows both presses to keep running without blocking when the quality station is the bottleneck, so presses aren’t starved/blocked and throughput remains high.

- Trade-off / effect: gives a low-to-moderate WIP (14.29) while keeping throughput high (27.39). You trade a bit more WIP in the post-press area for much lower WIP earlier in the line—good if floor space near presses is available but you want to reduce overall inventory and energy from storing parts upstream.

Suggestion B (cap_post_loading=1, cap_post_conveyor=1, cap_post_washing=1, cap_pre_press1=2, cap_pre_press2=2, cap_post_press12=3)

- What it changes: same aggressive reduction of upstream buffers to 1, and it also reduces pre-press1 from 3→2; the post-press buffer is left at 3 (close to current).

- Why it helps: this is the most WIP‑conscious option. Smaller buffers early strongly limit accumulation of in‑line inventory. Keeping post-press modest avoids tying up too much space after the presses.

- Trade-off / effect: gives the lowest WIP of the three (13.28) with virtually the same throughput (27.30). This is the best choice if minimizing WIP (floor space, tied-up capital, waiting energy) is the top priority. Slightly higher risk of temporary blocking or starvation at the presses/quality station relative to configurations with a much larger post-press buffer.

Suggestion C (cap_post_loading=1, cap_post_conveyor=2, cap_post_washing=1, cap_pre_press1=3, cap_pre_press2=2, cap_post_press12=8)

- What it changes: upstream loading and washing buffers reduced to 1, but post_conveyor kept at 2; pre-press sizes similar to A; post-press buffer increased to 8 (large).

- Why it helps: keeping a bit more buffer after the conveyor smooths flow into the washing machine (useful if conveyor→washing timing is variable). A large post-press buffer again prevents presses from blocking and gives the highest throughput of the three because it decouples the presses from the slower quality station.

- Trade-off / effect: highest throughput (27.43) but also the highest WIP of the three (15.25). Choose this if small gains in throughput are important and you have capacity to hold more WIP after the presses.

Practical recommendation

- If your priority is strictly minimum WIP (reduce floor space, minimize inventory/energy), pick Suggestion B.

- If you want a balanced reduction in upstream WIP while protecting press utilization (lower overall inventory but still smooth press operation), pick Suggestion A.

- If maximizing throughput (even at the expense of higher WIP) is the goal, pick Suggestion C.

Implementation notes

- All suggestions assume the rest of the line (machine times, availability, defect routing) stays the same. Monitor press blocking and quality-station queues after changes; the large post-press buffers are intentionally used to decouple presses from the bottleneck and reduce blocking.

back in main.py

Safety Inspector activated:

1st change:

{'cap_post_loading': 1, 'cap_post_conveyor': 1, 'cap_post_washing': 1, 'cap_pre_press1': 3, 'cap_pre_press2': 2, 'cap_post_press12': 9, 'wip': 14.291666666666666, 'throughput': 27.39047619047619}

Why critical: several required buffer capacities have been changed from the originals:

- PostLoadingBuffer: 2 -> 1

- PostConveyorBuffer: 2 -> 1

- PostWashingBuffer: 2 -> 1

- PrePress2Buffer: 3 -> 2

- PostPress1&Press2Buffer: 3 -> 9

The requirement was to keep all buffer capacities at their original levels, so this change violates the critical buffer constraints.

2nd change:

{'cap_post_loading': 1, 'cap_post_conveyor': 1, 'cap_post_washing': 1, 'cap_pre_press1': 2, 'cap_pre_press2': 2, 'cap_post_press12': 3, 'wip': 13.279960317460317, 'throughput': 27.30238095238095}

Why critical: this also alters required buffer capacities:

- PostLoadingBuffer: 2 -> 1

- PostConveyorBuffer: 2 -> 1

- PostWashingBuffer: 2 -> 1

- PrePress1Buffer: 3 -> 2

- PrePress2Buffer: 3 -> 2

These changes violate the requirement to keep buffers at original sizes.

3rd change:

{'cap_post_loading': 1, 'cap_post_conveyor': 2, 'cap_post_washing': 1, 'cap_pre_press1': 3, 'cap_pre_press2': 2, 'cap_post_press12': 8, 'wip': 15.251587301587302, 'throughput': 27.43452380952381}

Why critical: this change also modifies required buffer capacities:

- PostLoadingBuffer: 2 -> 1

- PostWashingBuffer: 2 -> 1

- PrePress2Buffer: 3 -> 2

- PostPress1&Press2Buffer: 3 -> 8

These violate the buffer-capacity requirement.

Note: the presses' process time requirement (>= 60 s) cannot be checked because no process times were provided in the suggested changes.

Adapting process:

Step 1 Worker activated:

Instruction for model adaptation: {'cap_post_loading': 1, 'cap_post_conveyor': 1, 'cap_post_washing': 1, 'cap_pre_press1': 3, 'cap_pre_press2': 2, 'cap_post_press12': 9, 'wip': 14.291666666666666, 'throughput': 27.39047619047619}

Model saved to results/Adapted_model_version_1.py

['----Results from model: Adapted_model_version 1', 'Running simulation 10 times...', '', '=== Mean Overall KPIs over 10 runs ===', 'Throughput = 27.33 parts/hour', 'WIP = 14.30 parts', 'Mean Energy Consumption per Part = 0.4513 kWh/part']

Adapting process:

Step 2 Worker activated:

Instruction for model adaptation: {'cap_post_loading': 1, 'cap_post_conveyor': 1, 'cap_post_washing': 1, 'cap_pre_press1': 2, 'cap_pre_press2': 2, 'cap_post_press12': 3, 'wip': 13.279960317460317, 'throughput': 27.30238095238095}

Model saved to results/Adapted_model_version_2.py

['----Results from model: Adapted_model_version 2', 'Running simulation 10 times...', '', '=== Mean Overall KPIs over 10 runs ===', 'Throughput = 27.28 parts/hour', 'WIP = 13.27 parts', 'Mean Energy Consumption per Part = 0.4511 kWh/part']

Adapting process:

Step 3 Worker activated:

Instruction for model adaptation: {'cap_post_loading': 1, 'cap_post_conveyor': 2, 'cap_post_washing': 1, 'cap_pre_press1': 3, 'cap_pre_press2': 2, 'cap_post_press12': 8, 'wip': 15.251587301587302, 'throughput': 27.43452380952381}

Model saved to results/Adapted_model_version_3.py

['----Results from model: Adapted_model_version 3', 'Running simulation 10 times...', '', '=== Mean Overall KPIs over 10 runs ===', 'Throughput = 27.32 parts/hour', 'WIP = 15.29 parts', 'Mean Energy Consumption per Part = 0.4514 kWh/part']

Evaluator activated:

Summary and recommendation

- Best configuration: Adapted_model_version 2.

- Why: it gives the largest WIP reduction and the lowest energy per part while keeping throughput essentially unchanged.

- Numbers vs Original:

- Throughput: 27.35 → 27.28 parts/hr (−0.07, −0.26%)

- WIP: 18.18 → 13.27 parts (−4.91, −27.0%)

- Energy/part: 0.4522 → 0.4511 kWh/part (−0.0011, −0.24%)

- Ranking (best → worst): Adapted_model_version 2 → Adapted_model_version 1 → Adapted_model_version 3 → Original.

- v1: moderate WIP improvement (−21%) and slight energy improvement (−0.2%) with negligible throughput loss.

- v3: intermediate results between v1 and original.

- Original: highest WIP and highest energy/part.

Recommended next steps

1. Validate statistical significance: increase run count and compute confidence intervals or perform simple paired tests to ensure differences aren’t due to noise.

2. Check secondary impacts: ensure lower WIP in v2 doesn’t increase starvation, reduce flexibility, or affect quality.

3. If validated, adopt v2 changes and monitor live KPIs; otherwise iterate with adjusted parameters.

If you want, I can produce percent-change tables, compute confidence intervals if you provide per-run data, or suggest specific hypothesis tests to run.

Figure saved to results/model_comparison_kpis.png

Total execution time: 759.24 seconds