cluster_env) [viktor@n213 Extension_LLM-DES-MOO-Modeling]$ python3 main.py

MachineName  Avg_Time_per_part_s  Availability_%  MTTR_s  EnergyIdling  EnergyWorking

Conveyor belt                  6.0          100.00     1.0          0.00           0.00

Hantering cell                 25.0           97.79    74.0          0.50           0.74

Loading robot                 12.0           90.49    68.0          0.25           0.72

Presses cell 1                175.0           87.79    73.0          1.25           1.28

Presses cell 2                176.0           87.69    74.0          1.25           1.27

Quality station cell                 41.0           85.87    66.0          0.58           0.84

Washing machine                 14.0           80.89   269.0          4.28          35.24

Directly-follows relationships:

Conveyor belt -> Washing machine

Loading robot -> Conveyor belt

Washing machine -> Hantering cell

Presses cell 2 -> Quality station cell

Hantering cell -> Presses cell 1

Parallel relationships:

Presses cell 1 || Presses cell 2

Presses cell 2 || Presses cell 1

Builder activated:

Inspector activated:

Building initial model based on the blueprint and input data...

Model saved to results/initial_model.py

Inspecting initial model for errors...

Inspecting code (Attempt 1)...

Run successful! The code is without errors.

Model saved to results/initial_model.py

Visualizing initial model into flow chart...

Visualizer activated:

Flow chart saved to: results/model_visualization.png

Evaluating initial model...

['----Results from model: Original model', 'Running simulation 10 times...', '', '=== Mean Overall KPIs over 10 runs ===', 'Throughput = 27.37 parts/hour', 'WIP = 18.09 parts', 'Mean Energy Consumption per Part = 0.4522 kWh/part']

=================================

---OPTIONAL OPTIMIZATION STAGE---

=================================

Optimizer activated:

==============================

-OPTIMIZATION INITIALIZED-

---OBJECTIVE SELECTION---

==============================

Please choose one of the following optimization scenarios:

1. WIP (Min) vs. Throughput (Max)

2. WIP (Min) vs. Energy Consumption per part (Min)

3. Throughput (Max) vs. Energy Consumption per part (Min)

[CONFIRMED] Scenario 1 loaded:

-> Objective 1: WIP (MIN)

-> Objective 2: THROUGHPUT (MAX)

------------------------------

================================

---DECISION VARIABLES SELECTION---

================================

Please choose one of the following approaches to optimizing your selected objectives:

1. Find the best buffer configuration

2. Decrease machine process time by som chosen percentage

3. Increase machine availability by some chosen percentage

4. Decrease machine MTTR by some chosen percentage

[CONFIRMED] Scenario 1 loaded:

The MOO algorithm will use these input variables to optimize the objectives:

All buffer capacities. Discrete values on the range 1-10

------------------------------

=================================

---HYPER PARAMETER SELECTION---

=================================

------------------------------

Generating MOO code...

Model saved to results/MOO_initial_code.py

Combining the MOO code with the simulation code...

Model saved to results/initial_combined_code.py

Generating a vizualization of the MOO algorithm...

Inspecting MOO and simulation code (Attempt 1)...

Model saved to results/checked_initial_combined_code.py

Running the MOO algorithm...

Run successful! Time taken: 248.26 seconds

See the results of the MOO algorithm in 'moo_simulation_results.csv'

And the pareto optimal solutions here: 'moo_pareto_solutions.csv'

Generating suggestions for improvements based on the Pareto-optimal solutions and user input...

The suggested datapoints for adaption can be seen in: results/'suggested_improvements.csv'

Suggested changes based on the MOO algorithm

{'instructions': [{'cap_post_loading': 1, 'cap_post_conveyor': 1, 'cap_post_washing': 1, 'cap_pre_press1': 3, 'cap_pre_press2': 2, 'cap_post_press12': 3, 'wip': 14.204563492063492, 'throughput': 27.30119047619048}, {'cap_post_loading': 1, 'cap_post_conveyor': 1, 'cap_post_washing': 1, 'cap_pre_press1': 3, 'cap_pre_press2': 3, 'cap_post_press12': 5, 'wip': 15.280555555555557, 'throughput': 27.41904761904762}, {'cap_post_loading': 1, 'cap_post_conveyor': 4, 'cap_post_washing': 1, 'cap_pre_press1': 3, 'cap_pre_press2': 6, 'cap_post_press12': 4, 'wip': 21.306150793650797, 'throughput': 27.529761904761905}]}

Below are short, manager-friendly explanations for each MOO suggestion. Each explanation says what changed, why it helps (how it affects starvation/blocking and flows), the trade-off you should expect, and when to prefer that setting.

1) Low‑WIP option — cap_post_loading=1, cap_post_conveyor=1, cap_post_washing=1, cap_pre_press1=3, cap_pre_press2=2, cap_post_press12=3

- What changed: Most delay buffers before the presses are reduced to size 1 (down from 2), pre_press2 lowered to 2; post-press buffer stays moderate (3).

- Why it helps: Smaller upstream buffers force tighter hand‑offs between stations, which reduces the average number of parts sitting between stages (WIP drops to ~14.2). Less WIP reduces inventory, storage space, and holding costs and makes process problems show up earlier.

- Trade-off / risk: Throughput is slightly lower than the throughput‑maximizing options (27.30 parts/hr) and the line becomes more sensitive to random breakdowns — short buffers can increase blocking/starvation episodes if a machine fails.

- When to use: When minimizing WIP, floor space and inventory cost is the priority and you can accept a small throughput hit and stricter maintenance discipline.

2) Balanced option — cap_post_loading=1, cap_post_conveyor=1, cap_post_washing=1, cap_pre_press1=3, cap_pre_press2=3, cap_post_press12=5

- What changed: Upstream buffers remain small (1) but the post‑press merge buffer is increased to 5.

- Why it helps: Keeping small upstream buffers keeps WIP low, while a larger post‑press buffer decouples the two presses from the quality station. That reduces blocking at the presses and slightly increases throughput (to ~27.42 parts/hr) without letting WIP grow much (WIP ≈ 15.3).

- Trade-off / risk: Slightly higher WIP than option 1, but improved stability and higher throughput. Requires a bit more post‑press storage space.

- When to use: When you want a good compromise — keep WIP low but recover some throughput and robustness at the press → quality handoff.

3) Throughput‑focused option — cap_post_loading=1, cap_post_conveyor=4, cap_post_washing=1, cap_pre_press1=3, cap_pre_press2=6, cap_post_press12=4

- What changed: Pre_press2 and post_conveyor buffers are enlarged (pre_press2 up to 6, post_conveyor to 4) and post_press12 moderately larger (4).

- Why it helps: Bigger buffers in front of the presses and after the conveyor decouple stages so the presses can run continuously despite upstream variability or short breakdowns. That yields the highest throughput of the three (~27.53 parts/hr).

- Trade-off / risk: WIP increases noticeably (≈21.3), which means more in‑process inventory, space, and higher holding costs. Larger buffers also mask problems longer.

- When to use: When maximizing output is the main goal and you accept higher WIP and inventory costs to keep the presses fed and running.

Summary recommendation

- If you must minimize WIP/costs: pick option 1.

- If you want a balance between low WIP and good throughput/robustness: pick option 2.

- If you must maximize throughput and can tolerate more WIP: pick option 3.

All three keep the loading side small (cap_post_loading=1) because the MOO found low upstream buffers help reduce overall WIP; the differences are primarily about how much decoupling to give the press/quality section.

back in main.py

Safety Inspector activated:

1st change: {'cap_post_loading': 1, 'cap_post_conveyor': 1, 'cap_post_washing': 1, 'cap_pre_press1': 3, 'cap_pre_press2': 2, 'cap_post_press12': 3, 'wip': 14.204563492063492, 'throughput': 27.30119047619048}

- Why critical: PostLoading, PostConveyor and PostWashing buffers are set to 1 (original = 2) and PrePress2 is set to 2 (original = 3). The requirement states all buffer capacities must be kept at their original levels, so this change violates the critical-buffer constraint.

2nd change: {'cap_post_loading': 1, 'cap_post_conveyor': 1, 'cap_post_washing': 1, 'cap_pre_press1': 3, 'cap_pre_press2': 3, 'cap_post_press12': 5, 'wip': 15.280555555555557, 'throughput': 27.41904761904762}

- Why critical: PostLoading, PostConveyor and PostWashing buffers are set to 1 (original = 2) and PostPress1&2 buffer is increased to 5 (original = 3). Changing these buffer capacities violates the critical-buffer constraint.

3rd change: {'cap_post_loading': 1, 'cap_post_conveyor': 4, 'cap_post_washing': 1, 'cap_pre_press1': 3, 'cap_pre_press2': 6, 'cap_post_press12': 4, 'wip': 21.306150793650797, 'throughput': 27.529761904761905}

- Why critical: PostLoading and PostWashing are set to 1 (original = 2); PostConveyor is set to 4 (original = 2); PrePress2 is set to 6 (original = 3); PostPress1&2 is set to 4 (original = 3). Multiple buffers differ from their required original capacities, so this change violates the critical-buffer constraint.

Note: none of the changes specify press process times; the "presses need to have a processtime of at least 60s" constraint is not addressed by these entries (no processtime field present).

Adapting process:

Step 1 Worker activated:

Instruction for model adaptation: {'cap_post_loading': 1, 'cap_post_conveyor': 1, 'cap_post_washing': 1, 'cap_pre_press1': 3, 'cap_pre_press2': 2, 'cap_post_press12': 3, 'wip': 14.204563492063492, 'throughput': 27.30119047619048}

Model saved to results/Adapted_model_version_1.py

['----Results from model: Adapted_model_version 1', 'Running simulation 10 times...', '', '=== Mean Overall KPIs over 10 runs ===', 'Throughput = 27.26 parts/hour', 'WIP = 14.25 parts', 'Mean Energy Consumption per Part = 0.4537 kWh/part']

Adapting process:

Step 2 Worker activated:

Instruction for model adaptation: {'cap_post_loading': 1, 'cap_post_conveyor': 1, 'cap_post_washing': 1, 'cap_pre_press1': 3, 'cap_pre_press2': 3, 'cap_post_press12': 5, 'wip': 15.280555555555557, 'throughput': 27.41904761904762}

Model saved to results/Adapted_model_version_2.py

['----Results from model: Adapted_model_version 2', 'Running simulation 10 times...', '', '=== Mean Overall KPIs over 10 runs ===', 'Throughput = 27.31 parts/hour', 'WIP = 15.21 parts', 'Mean Energy Consumption per Part = 0.4523 kWh/part']

Adapting process:

Step 3 Worker activated:

Instruction for model adaptation: {'cap_post_loading': 1, 'cap_post_conveyor': 4, 'cap_post_washing': 1, 'cap_pre_press1': 3, 'cap_pre_press2': 6, 'cap_post_press12': 4, 'wip': 21.306150793650797, 'throughput': 27.529761904761905}

Model saved to results/Adapted_model_version_3.py

['----Results from model: Adapted_model_version 3', 'Running simulation 10 times...', '', '=== Mean Overall KPIs over 10 runs ===', 'Throughput = 27.28 parts/hour', 'WIP = 21.27 parts', 'Mean Energy Consumption per Part = 0.4537 kWh/part']

Evaluator activated:

Summary of results (10-run means)

- Original

- Throughput = 27.37 parts/hr (highest)

- WIP = 18.09 parts

- Energy = 0.4522 kWh/part (lowest)

- Adapted_model_version 1

- Throughput = 27.26 (-0.11 vs Original)

- WIP = 14.25 (-3.84 vs Original — lowest WIP)

- Energy = 0.4537 (+0.0015 vs Original)

- Adapted_model_version 2

- Throughput = 27.31 (-0.06 vs Original)

- WIP = 15.21 (-2.88 vs Original)

- Energy = 0.4523 (+0.0001 vs Original)

- Adapted_model_version 3

- Throughput = 27.28 (-0.09 vs Original)

- WIP = 21.27 (+3.18 vs Original — highest)

- Energy = 0.4537 (+0.0015 vs Original)

Which configuration is "best"

- If your primary objective is maximizing throughput (possibly with minimal energy), the Original model is best: it has the highest throughput and the lowest energy per part.

- If your primary objective is minimizing WIP (fewer parts in system), Adapted_model_version 1 is best: it reduces WIP substantially (≈3.8 parts) at the cost of a very small throughput drop (0.11 parts/hr) and a negligible energy increase (~0.0015 kWh/part).

- Adapted_model_version 2 is the best compromise if you want lower WIP than Original with almost no loss in throughput and essentially the same energy.

- Adapted_model_version 3 is the weakest choice (highest WIP and no throughput/energy advantage).

Recommendation and next steps

1. Choose based on your priority:

- Throughput/energy priority → keep Original.

- WIP priority → choose Adapted_model_version 1.

- Balanced trade-off → choose Adapted_model_version 2.

2. Validate statistical significance: run more replications and compute confidence intervals (or perform hypothesis tests) — the observed differences are small and may be within simulation noise.

3. If you need an objective aggregate, define weights for throughput/WIP/energy and compute a weighted score or construct a Pareto front to make a formal multi-objective decision.

If you tell me which KPI(s) you care about most (throughput, WIP, energy, or a weighted combination), I can pick the single best configuration and/or compute a simple weighted ranking.

Figure saved to results/model_comparison_kpis.png

Total execution time: 791.17 seconds